Study Material

| <Program Code: CO/CM/CW/DE/EJ/ET/EN/EX/EQ/IE/IS/IC/MU>: <Course Code:22320><Course Name: Digital Techniques><Topic Name: Number Systems and code> <UO1b.2.><study Material> |  |  |
| --- | --- | --- |
| Course Expert: Mrs Rita Vora | Date:14/07/2020 | Mentor: Mr. K. P. Akole Co-ordinator: Mrs Nirmala Kamble |
| Key words 1's complement subtraction | Learning Objective:. To apply 1's complement method for subtraction |  |
| Key Questions Q1. Define 1's complement Q2. What is the first thing you must do in order to solve the subtraction problem (11)10 – (4)10 using 1's complement representation? | Concept Map |  |
|  | A ones' complement system or ones' complement arithmetic is a system in which negative numbers are represented by the inverse of the binary representations of their corresponding positive numbers. In such a system, a number is negated (converted from positive to negative or vice versa) by computing its ones' complement. An N-bit ones' complement numeral system can only represent integers in the range −(2N−1−1) to 2N−1−1 . | Key Definitions 1’s complement : The 1’s complement of a binary number is defined as the value obtained by inverting all the bits in the binary representation of the number (swapping 0s for 1s and vice versa). |
| Solved word Problem . Q2. Subtract (1110)2- (1001)2 using 1's complement. Solution: 1’s complement of i)(1001)2 is (0110)2. ii)(1110)2 +(0110)2= 1 0100. iii) Carry is 1 so add to 0100= 0101 So Answer is (0101)2 |  |  |
|  | Application of 1’s complement method - Computers - Adders - Subtractors - Digital Systems | Link to YouTube/ OER/ video https://www.youtube.com/watch?v=kx8DFNf6pQU |

1’s Complement of a Binary Number: - The 1’s complement of a binary number is important because they can be used for representation of negative numbers. - The 1’s complement of a number can be obtained simply by changing all 1’s to 0’s and 0’s to 1’s. - The I’s complement of a number is found by inverting all the bits in that number. This is called as taking complement. - This complemented value represents the negative of the original number. The 1’s complement system is very easy to implement using inverters. Example: Find 1’s complement of the given binary numbers. a) 101010 b)110101 Solution: a)101010 b)110101 ↓↓↓↓↓↓ ↓↓↓↓↓↓ 010101 001010 There is a simple algorithm to convert a binary number into 1’s complement. To get 1’s complement of a binary number, simply invert the given number. You can simply implement logic circuit using only NOT gate for each bit of Binary number input. Implementation of logic circuit of 4-bit 1’s complement is given as following below. Binary number subtraction using 1’s complement can be performed as follows: The steps to be followed for subtraction of (A)2-(B)2 where A is the minuend and B is the subtrahend using 1’s complement are: Step 1: Find 1’s complement of the subtrahend B. Step 2: Add 1’s complement of B to A. Step 3: If final carry is 1 then add it to the result of addition obtained in step 2. The carry is called end around carry. If carry is 1, then the result of subtraction is (+ve) and in its true form. Step 4: If the carry produced in step 2 is 0, then the result obtained is (–ve) and in 1’s complement form, so convert it into true form. Examples where a smaller number is to be subtracted from a larger number: i)Perform (9)10 – (4)10 using 1’s complement method. Solution; (9)10 = (1001)2 (4)10 = (0100)2 Step 1: 1’s complement of (0100)2 =(1011)2 Step 2: Add (9)10 and1’s complement of (4)10 Minuend- 1 0 0 1 1’s complement of subtrahend 1 0 1 1 Carry over - 1 0 1 0 0 Step 3: Add the final carry + 1 0 1 0 1 Answer is positive and in true form The Answer is (0101)2 (iI) (110101)2 – (100101)2 Step 1: 1’s complement of (10011)2is ( 011010)2. Step 2: Minued - 1 1 0 1 0 1 1’s complement of subtrahend - + 0 1 1 0 1 0 Carry over 1 0 0 1 1 1 1 Step 3: Add the final carry + 1 0 1 0 0 0 0 Answer is positive and in true form The Answer is (010000)2 ii) (1011.001)2 – (110.10)2 Solution: Step 1: 1’s complement of (0110.100)2 is (1001.011)2 Step 2: Minued - 1 0 1 1 . 0 0 1 1’s complement of subtrahend - 1 0 0 1 . 0 1 1 Carry over 1 0 1 0 0 . 1 0 0 Step 3: Add the final carry + 1 0 1 0 0 . 1 0 1 Answer is positive and in true form The Answer is (0100 .101)2 Examples where a larger number is to be subtracted from a smaller number: i ) (101011)2 – (111001)2 Solution: Step 1: 1’s complement of (111001)2 is (000110)2. Hence Step 2: Minued - 1 0 1 0 1 1 1’s complement - 0 0 0 1 1 0 Carry is 0 1 1 0 0 0 1 Step 3: Final carry is 0 so answer is negative and in 1’s complement form So final answer is -(0011100)2 (1’s complement of (110001)2) ii) (10110.01)2 – (11010.10)2 Solution: Step 1: 1’s complement of (11010.10)2 is (00101.01)2 Step 2: Minuend 1 0 1 1 0 . 0 1 1;s complement 0 0 1 0 1 . 0 1 Carry is 0 1 1 0 1 1 . 1 0 Step 3:Final carry is 0 so answer is negative and in 1’s Complement form So final answer is -(00100.01)2 (1’s complement of (11011.10)2)

Key Take away: 1’s complement method can be used for subtraction of two numbers in corresponding number systems and is simpler than Binary Subtraction.